# ;;;;The Internet Extender Users Manual

# System Requirements

- IBM/Compatible PC with a 286 or higher processor

- 1MB of RAM

- <1MB or hard drive space or 1 bootable floppy

- MSDOS 5.0 or higher

- VGA adapter

- 14.4Kbps or higher modem (if you use a modem to connect to the Internet)

- Network Interface Card + Packet Driver (2 if you don’t use a modem to connect)

- PPP packet driver (EPPPD for DOS –freeware- recommended)

# Installation

Running the Internet Extender from the Hard Disk:

Uncompress the Internet Extender Archive into a temporary directory and run the

SETUP.BAT script.  The script will create an INET_EXT directory on your hard drive and copy the necessary files to that directory depending on how you answer the questions it asks.  There are 2 main configurations of the software:

- You are using a modem to dial up your ISP to dynamically receive an IP address and connection to the Internet.  You then have an Ethernet Adapter connected to your local area network with its own addressing scheme.

- You are using an Ethernet Adapter to connect through a Cable Modem, T1, etc. and your IP address is statically assigned to you.  You then also have an Ethernet Adapter connected to your local area Network with its own addressing scheme.

Note:  If you are using DHCP on your LAN, you will need to exclude an address and statically assign it to the Internet Extender Machine.

The SETUP.BAT file is not perfect so I would suggest checking the configuration files that it creates before trying to run the program.  There are 2 main configuration files that the Internet Extender needs to run: NAPT.INI and PPPD.CFG (for modems only).  I will explain each file in detail so that you will know exactly what information you will need to get before running the program.  All of the configuration files should reside in the same directory as the Internet Extender executable.

Note:  The program comes with about 10 packet drivers for Ethernet Adapters.  This in no way will cover every Adapter made, so if the packet driver specific to your Adapter isn’t included you will need to get it from the Installation Disk that your Adapter came with, from the adapter website, or ftp://ftp.crynwr.com

## NAPT.INI

local_ip=10.1.1.100

local_mask=255.0.0.0

local_gateway=10.1.1.254

remote_ip=204.182.120.203

remote_mask=255.255.253.0

remote_gateway=204.182.124.254

local_pdi=0x61

remote_pdi=0x60

tcp_server=10.1.1.9:21

udp_server=10.1.1.4:6667

local_ip:  	You specify the IP address of the Ethernet Card connected to your local

area network.  It must be static, the program does not use DHCP.

local_mask:	You specify the Subnet Mask specific to your local area network.

remote_ip:	You specify the IP address of the Ethernet Card connected ultimately to

the Internet.  **if using a modem, place a semicolon before remote_ip.

remote_mask:	You specify the Subnet Mask specific to the ISP’s addressing scheme.

**if using a modem, place a semicolon before remote_mask.

local_pdi:	You specify the packet driver interrupt (pdi) of the Ethernet Card

connected to your local area network.  **must be between 0x60-0x80.

remote_pdi:	You specify the packet driver interrupt (pdi) of the Ethernet Card/Modem

connected to the Internet.  **PPPD will grab the first available interrupt.

tcp_server:	You specify the IP address:Port of any TCP servers that reside on the

local area network that you would like people on the Internet to have

access to.  You can have as many as you wish, but they must have there

own line for each one.  Place a semicolon before tcp_server if you do not

have any servers you would like world access to.

udp_server:	You specify the IP address:port of any UDP servers that reside on the

local area network.  Works the same as the tcp_server entry.

local_gateway:	You specify the IP address of the local gateway if one exists that you want

to include.  **place a semicolon before local_gateway if not specified.

remote_gateway:  You specify the IP address of the remote gateway if it not automatically

assigned by the ISP.

**place a semicolon before remote_gateway if not	specified.

PPPD.CFG  	**for use with a modem only**

COM2

57600

modem

connect "chat -e '' ATDT5551212 login: username word: password"

COM#:	You specify which COM port your modem resides on.

57600:		You specify the maximum speed at which your modem can connect.

modem:	You are using a modem to connect.

connect:	It calls the chat tool to call and connect to the ISP to receive the

addressing information.  You must specify the phone number of the ISP

by replacing 5551212 with your number.  You must specify your

username/password by replacing them respectively with your own.

Note:  For more information about the PPPD.CFG or any part of the PPP driver configuration, view the readme.txt included with the Internet Extender.  It was written by Antonio Lopez Molero and his readme.txt will give you more insight as to the options that can be included.

The final thing that you must worry about is how to execute the program.  These are the final steps in running the program.  At the DOS prompt:

**The name of the Ethernet Packet Driver, and an Interrupt**

ex. 3C5X9PD.COM 0x61

**The name of your PPP Packet Driver, and an Interrupt**

ex. For a second Ethernet Card  3C5X9PD.COM 0x60

ex. For a modem EPPPD.EXE (interrupt automatically set to first available)

NAPT.EXE  **Runs the Internet Extender Program**

OR just run the INET_EXT.BAT file that SETUP created to run the program.

The packet drivers must be loaded before the program is run or else the program will not start.  It will inform you which packet driver has not been properly loaded beforehand.

Running the Internet Extender from Floppy:

Create a bootable floppy with your Operating System (DOS/Windows) then

unzip the program archive to your hard drive into any temporary directory.

Run SETUP.BAT from the hard disk and when it asks you where you want it

installed, specify the floppy drive.  But remember that you need to make sure

that you have the packet driver for your specific Network Adapter on the disk

as well.  The process is the same as running from the Hard Disk.

Configuring for Local Servers

The Internet Extender is also designed to support static mappings of TCP and UDP servers.  It allows for Servers on the Local Area Network with invalid IP addresses to be accessible by the global Internet.  To accomplish this, you must know the local IP address and port number the server listens on.  Once that information is found, you only need to add a line to the NAPT.INI file stating whether it is a TCP server or a UDP server.  The line is as follows:

tcp_server=<IP address>:<port>  ex. 10.64.1.1:21  *usually an ftp server on 21

udp_server=<IP address>:<port>  ex. 10.64.1.2:6667  *IRC server on 6667

And as you add servers, just add another line stating whether it is a TCP or UDP server and the IP address and port information.  The way the Internet Extender will use this information, is it will set aside the port numbers you specified and when an incoming request comes to the Globally unique IP address given by your ISP, and the port number matches, it will map that request to the server machine it references.  The only limitation is that you cannot have more than one server using the same port number.  If your servers are configurable and you have two of the same type (ie. FTP servers), then you will have to change the port number that one of the servers listens on for them to both function properly.

How to Access the Internet with the Internet Extender

The way that the program is designed to function is as a gateway to the Internet.  The machine is multi-homed, meaning either 2 Network Interface Cards or a Network Interface Card and a Modem.  Each device is connected to a separate network with its own addressing scheme.  Each network also only sees the card and respective IP address that it is physically connected to.  The software is designed to do the translation between the two separate networks.  The machines on the Local Area Network should treat the Internet Extender as their Gateway to the Internet.

In order for this to happen, each machine needs to be simply configured to direct all traffic not intended for the Local Area Network to the Internet Extender.  This is done by telling each machine that the Gateway address is the local IP address that was statically assigned to the Internet Extender.  The program will receive all requests from the local network, translate the addressing information and forward it onto the Internet and vice-versa.  So the only configuration necessary for each local client machine is the gateway address which is the local IP address of the Internet Extender.

Credits

I would like to thank Dr. Dan Lewis, Head of the Computer Engineering Department at Santa Clara University, for his countless hours of work that he put into me and the program itself.  Without his expertise and help, the program would not have anywhere near the functionality that it does at this point.  I appreciate everything that he has done for me as a student, and everything he has done on the program itself.  Thank you.

Lastly, I would like to thank Antonio Lopez Molero for writing the PPPD for DOS packet driver and the people that contributed to its success.  The PPP driver has worked flawlessly and it has contributed to the success of the program because I can count on it functioning properly.  You can visit the website for the PPPD program at:

http://mvmpc9.ciw.uni-karlsruhe.de:80/c:/user/toni/

You can find packet drivers for your specific Ethernet Cards at their respective manufacturer’s websites or at the Crynwr ftp server:

ftp://ftp.crynwr.com

You can visit the Internet Extender’s website for updates and troubleshooting tips at:

http://www-acc.scu.edu/~jsarich/ieweb